STM32-Based Dino Game with OLED Display and Smart Features

📖 Project Description

This project is a real-time embedded implementation of the classic Chrome Dino game, developed using an STM32 microcontroller. The system integrates hardware peripherals such as an OLED display, push buttons, touch sensor, buzzer, and rotary encoder to create an interactive gaming experience.

The game is built using a structured state-machine architecture, enabling smooth transitions between menu, level selection, gameplay, and game-over states. It incorporates real-time physics, collision detection, dynamic difficulty scaling, and persistent high-score storage using internal Flash memory.

The project demonstrates practical application of embedded systems concepts including GPIO interfacing, PWM signal generation, I2C communication, interrupt handling, and memory management.

🎮 Features

- Multi-level gameplay (Easy, Medium, Hard)

- Smooth character animation with gravity and jump physics

- Dynamic obstacle and enemy (bird) movement

- Touch-activated shield mechanism with time-based control

- Real-time sound effects using PWM (Timer-based buzzer control)

- High score storage in Flash memory (non-volatile)

- Rotary encoder-based speed control

- OLED display (SSD1306) for graphics rendering

- State-machine based game logic

🔌 Hardware Components

- STM32F103C8T6 (Blue Pill)

- SSD1306 OLED Display (128x64, I2C)

- Push Button (Game control)

- Touch Sensor (Shield activation)

- Buzzer (PWM sound output)

- Rotary Encoder (Speed adjustment)

- Lithium Battery + TP4056 Charging Module

- DC-DC Boost Converter

🔗 Hardware Connections

🖥 OLED Display (I2C)

- VCC → 3.3V

- GND → GND

- SCL → PB6

- SDA → PB7

🔘 Push Button

- One terminal → PA1

- Other terminal → GND

- Configured as INPUT with PULL-UP

👆 Touch Sensor

- Output → PA5

- VCC → 3.3V

- GND → GND

🔊 Buzzer (PWM Output)

- Positive → PA8 (TIM1 Channel 1)

- Negative → GND

🎛 Rotary Encoder

- CLK → PB10 (Interrupt pin)

- DT → PB11

- VCC → 3.3V

- GND → GND

🔋 Power System

- Lithium Battery → TP4056 Charging Module

- Output → Boost Converter (5V)

- Supply to STM32 via 5V pin

⚙️ Software Implementation

🧠 Architecture

- Finite State Machine:

  - MENU

  - LEVEL SELECTION

  - GAME PLAY

  - GAME OVER

🔄 Core Functionalities

- Real-time game loop with frame updates (~30ms delay)

- Gravity and jump physics simulation

- Collision detection using bounding box logic

- Obstacle randomization and spacing logic

- Dynamic speed increment over time

- Debounced input handling (button & touch sensor)

💾 Memory Handling

- High score stored in internal Flash memory

- Flash erase + write using HAL APIs

🔊 Sound System

- PWM-based tone generation using Timer1

- Different tones for:

  - Jump

  - Game Over

  - Shield activation

  - Background effects

🛠 Technical Skills Demonstrated

🔹 Embedded Systems

- STM32 HAL drivers

- Peripheral configuration (GPIO, I2C, TIM, ADC)

- Real-time system design

🔹 Hardware Interfacing

- Sensor integration (touch sensor, encoder)

- Display interfacing (SSD1306 via I2C)

- PWM signal generation

🔹 Firmware Development

- Embedded C programming

- State-machine design pattern

- Timing and delay optimization

🔹 Signal Processing & Control

- Debouncing techniques

- Interrupt-based input handling

- PWM frequency modulation for audio

🔹 Memory Management

- Flash memory read/write operations

- Persistent data storage

🔹 System Design

- Modular code structure

- Real-time event handling

- Game logic optimization for microcontrollers

🚀 Future Improvements

- Add Bluetooth/WiFi connectivity (ESP32 integration)

- Multiplayer mode

- Mobile app control

- Improved graphics and animations

- Power optimization for wearable version

👩‍💻 Author

Dhaneshwari Bhujade
Electronics Engineering Student